Installation of ROS2 Jazzy Jalisco in Linux Ubuntu 24.04 (Noble)

Contents

- Resources

- System setup

- Set locale

- Enable required repositories

- Install development tools

- Install ROS 2

- Setup environment

- Resources

Status Page:

- ROS 2 Jazzy (Ubuntu Noble 24.04): amd64, arm64

- Jenkins Instance

- Repositories

- System setup (Follow the commands one by one)

- Set locale

Locale     # Check for UTF-8

sudo apt update && sudo apt install locales

sudo locale-gen en_US en_US.UTF-8

sudo update-locale LC_ALL=en_US.UTF-8 LANG=en_US.UTF-8

export LANG=en_US.UTF-8

locale     # verify settings

- Enable required repositories

- First ensure that the Ubuntu Universe repository is enabled.

sudo apt install software-properties-common

sudo add-apt-repository universe

- Now add the ROS 2 GPG key with apt.

sudo apt update && sudo apt install curl -y

sudo curl -sSL  https://raw.githubusercontent.com/ros/rosdistro/master/ros.key -o /usr/share/keyrings/ros-archive-keyring.gpg

- Then add the repository to your sources list.

echo "deb [arch=$(dpkg --print-architecture) signed-by=/usr/share/keyrings/ros-archive-keyring.gpg] http://packages.ros.org/ros2/ubuntu $(. /etc/os-release && echo $UBUNTU_CODENAME) main" | sudo tee /etc/apt/sources.list.d/ros2.list > /dev/null

- Install development tools

sudo apt update && sudo apt install ros-dev tools

- Install ROS 2

- Update & Upgrade your apt repository caches after setting up the repositories.

sudo apt update

sudo apt upgrade

sudo apt install ros-jazzy-desktop

sudo apt install ros-jazzy-ros-base

- Setup Environment

source /opt/ros/jazzy/setup.bash

Result (Upto setting up of environment)

Installation and execution of turtlesim package

- Turtlesim is a lightweight simulator for learning ROS 2.

- It illustrates what ROS 2 does at the most basic level to give you an idea of what you will do with a real robot or a robot simulation later on.

- Install Turtlesim

start by sourcing your setup files in a new terminal

sudo apt update

sudo apt install ros-jazzy-turtlesim

Check whether the packages are installed

ros2 pkg executables turtlesim

- Start Turtlesim

ros2 run turtlesim turtlesim_node

- Use Turtlesim

Open a new terminal and source ROS 2 again.

ros2 run turtlesim turtle_teleop_key

Use the arrow keys on your keyboard to control the turtle.

- Results

(The display after the copmpletion of 2nd step)

(Display after the completion of 3rd step)

(Display of moved turtle in random directions)